** sch_path: /CS_resistive_load.sch

**.subckt CS_resistive_load

R1 Vdd vout {r} m=1

Vdd Vdd GND {vdd}

Vin Vin GND sin(1.65 1e-3 1e6 0 0 0) ac 1

XM2 vout Vin GND GND nfet_03v3 L={l1} W={w1} nf=1 ad='int((nf+1)/2) * W/nf * 0.18u' as='int((nf+2)/2) * W/nf * 0.18u' pd='2*int((nf+1)/2) * (W/nf + 0.18u)'

+ ps='2*int((nf+2)/2) * (W/nf + 0.18u)' nrd='0.18u / W' nrs='0.18u / W' sa=0 sb=0 sd=0 m={m}

C1 vout GND {c} m=1

**** begin user architecture code

.param r=3k

.param vdd=3.3

.param m=1

.param w1=10u

.param l1=1u

.param c=1p

.control

*dc vin 0 3.3 0.01

*tran 1n 5e-6

*let v_out = vecmax(v(vout))-vecmin(v(vout))

*let v_in = vecmax(vin) - vecmin(vin)

*let gain=db(v_out/v_in)

*print gain

ac dec 10 10 100e10

let gain_max=vecmax(vdb(vout))

let three_db_gain=gain_max-3

meas ac freq_three_db  WHEN vdb(vout)=three_db_gain

meas ac cutoff_freq WHEN vdb(vout)=gain_max

print gain_max

print three_db_gain

print cutoff_freq

*meas dc v_bias WHEN v(vin) = v(vout)

*save all

.endc

*.include /design.ngspice

*.lib /sm141064.ngspice typical

**** end user architecture code

**.ends

.GLOBAL GND

.end